# Kobra 2 Neo to AWS IoT Core: device spec

Oct 25, 2026 · @Deepak

## Overview

The Arduino UNO Q next to the printer publishes seven MQTT topics to AWS IoT Core: telemetry, vibration summaries and events every few seconds (buffered while offline), raw 400 Hz accelerometer data and camera snapshots (live only), plus retained setup and online status. It needs one thing, one certificate and one policy from you.

- Transport: MQTT 3.1.1 over TLS 1.2 on port 8883, authenticated by one X.509 device certificate. Client ID kobra2neo-unoq.

- Publish only: the board never subscribes, so nothing in AWS can send it commands. The printer link is read-only too (query G-codes only).

- Compatible with phase 1: schema v2 keeps every field of Deepak's PC gateway (printer_gateway_v1.py, schema v1) and adds more. The topics printer/kobra/telemetry and printer/kobra/status are unchanged.

- Region: the phase 1 config used eu-west-2 (London). Every ARN and the endpoint below assume it; change them if you use another region.

data path · board to AWS IoT Core

Data only flows left to right: AWS has no path back to the board or the printer.

## What to create in AWS

Create one thing with one active certificate, attach the policy in the next section, and send Deepak three files plus the endpoint. The AWS CLI steps below do it in a few minutes; the IoT Core console works the same way.

- Create the thing:

aws iot create-thing --thing-name kobra2neo-unoq

- Create an active certificate and key pair. Note the certificateArn it prints.

aws iot create-keys-and-certificate --set-as-active \
  --certificate-pem-outfile device.pem.crt \
  --private-key-outfile private.pem.key \
  --public-key-outfile public.pem.key

- Create the policy from policy.json (next section), then attach it to the certificate and the certificate to the thing:

aws iot create-policy --policy-name kobra2neo-unoq-policy --policy-document file://policy.json
aws iot attach-policy --policy-name kobra2neo-unoq-policy --target <certificateArn>
aws iot attach-thing-principal --thing-name kobra2neo-unoq --principal <certificateArn>

- Get the account's data endpoint (it ends in -ats.iot.eu-west-2.amazonaws.com):

aws iot describe-endpoint --endpoint-type iot:Data-ATS

- Download the Amazon root CA:

curl -o AmazonRootCA1.pem https://www.amazontrust.com/repository/AmazonRootCA1.pem

Hand to Deepak: AmazonRootCA1.pem, device.pem.crt, private.pem.key and the endpoint address. They go in the app's python/certs/ folder on the board. Send the private key over a private channel and don't keep extra copies; it can't be downloaded again. If the board is lost, deactivate the certificate in IoT Core.

## IoT policy (least privilege)

The certificate may connect only as kobra2neo-unoq and publish only under printer/kobra/. Retained publishing is limited to the two retained topics. There is no iot:Subscribe or iot:Receive, so nothing can reach the board through MQTT. Replace ACCOUNT_ID, and the region if it isn't eu-west-2.

{
  "Version": "2012-10-17",
  "Statement": [
    {
      "Effect": "Allow",
      "Action": "iot:Connect",
      "Resource": "arn:aws:iot:eu-west-2:ACCOUNT_ID:client/kobra2neo-unoq"
    },
    {
      "Effect": "Allow",
      "Action": "iot:Publish",
      "Resource": "arn:aws:iot:eu-west-2:ACCOUNT_ID:topic/printer/kobra/*"
    },
    {
      "Effect": "Allow",
      "Action": "iot:RetainPublish",
      "Resource": [
        "arn:aws:iot:eu-west-2:ACCOUNT_ID:topic/printer/kobra/status",
        "arn:aws:iot:eu-west-2:ACCOUNT_ID:topic/printer/kobra/setup"
      ]
    }
  ]
}

- The * in a topic ARN matches any characters, including /, so it also covers printer/kobra/snapshot/<unix_ms>.

- The Last Will (status = offline) is published by IoT Core on the board's behalf, so it is also checked against iot:Publish and iot:RetainPublish on printer/kobra/status. Both are covered above.

- Optional hardening: add the condition "Bool": {"iot:Connection.Thing.IsAttached": "true"} to the Connect statement. That works here because the client ID equals the thing name.

## Topics and volumes

About 262,000 messages a day reach IoT Core, two thirds of them raw accelerometer blocks. Sizes are from a test run against a local broker with the real code.

| Stream | Topic | Rate | Delivery | Typical size | Messages/day |
| --- | --- | --- | --- | --- | --- |
| Telemetry | printer/kobra/telemetry | every 2 s | QoS 1, buffered offline | 0.74 kB | 43,200 |
| Vibration summary | printer/kobra/vibration | every 2 s | QoS 1, buffered offline | 1.5 kB | 43,200 |
| Events | printer/kobra/event | when one happens | QoS 1, buffered offline | 0.2 kB | a few |
| Raw accelerometer | printer/kobra/accel_raw | 1 s per sensor | QoS 0, live only | 3.5 to 3.9 kB | 172,800 |
| Camera snapshot | printer/kobra/snapshot/<unix_ms> | every 30 s | QoS 0, live only | up to 120 kB JPEG | 2,880 |
| Setup and bed mesh | printer/kobra/setup | on connect or change | QoS 1, retained | 1.4 kB | a few |
| Online status | printer/kobra/status | on change, plus Last Will | QoS 1, retained | 0.2 kB | a few |

IoT Core meters messages in 5 KB steps, so a 100 kB snapshot counts as 20 messages. Raw data is about 600 MB of payload a day. If cost matters, the board can turn raw data off or send snapshots less often (Board side, below); check the current IoT Core pricing for your region.

## Payload formats

Every JSON message starts with the same envelope; snapshots are bare JPEG bytes. Values the printer hasn't reported are null. Temperatures are in °C, positions in mm, acceleration in g, times in UTC.

| Field | Meaning |
| --- | --- |
| v | schema version, 2 (phase 1 gateway = 1) |
| dev | device ID, kobra2neo |
| ts | when the board built the message, UTC ISO 8601 with ms |
| gw | board software, unoq-aws-1 |
| seq | on queued topics only: increasing message number (see Delivery) |

Telemetry (printer/kobra/telemetry). nozzle, bed and progress are the phase 1 fields. pwm is the heater duty out of 127, pwm_pct the same in %. pos.*_act is the motor position from step counts, z_corr the live mesh correction, and lead how far the planner runs ahead of the motors. sample_t is when the printer values were read.

{"v": 2, "dev": "kobra2neo", "ts": "2026-10-25T21:10:56.882Z", "gw": "unoq-aws-1", "seq": 1,
 "nozzle": {"act": 220.02, "tgt": 220.0, "pwm": 40.0, "pwm_pct": 31.5},
 "bed": {"act": 60.17, "tgt": 60.0, "pwm": 30.0, "pwm_pct": 23.6},
 "progress": {"printing": true, "bytes": 18813, "total": 248792, "pct": 7.56},
 "pos": {"x": 85.81, "y": 78.15, "z": 0.2, "e": 26.5, "x_act": 85.5, "y_act": 78.3375, "z_act": 0.03, "z_corr": -0.17, "lead": 0.362},
 "job": {"file": "00_JLR_specon_pin_v2_1pc_PLA_6m28s.gcode", "print_time_s": 31.0, "slicer_est_s": 388.0,
         "eta_slicer_s": 357, "eta_progress_s": 379, "speed_pct": 100.0, "flow_pct": 100.0, "filament_mm": 0.0, "filament_g": 0.0},
 "endstops": {"x_min": "open", "y_min": "open", "z_min": "TRIGGERED", "z_max": "open", "filament": "TRIGGERED"},
 "sample_t": "2026-10-25T21:10:56.820Z"}

Vibration summary (printer/kobra/vibration). One object per sensor: table (on the printer's table) and toolhead (on the extruder). Axis arrays are [x, y, z] in the sensor's own frame. Everything is computed over the last 2 s with the mean removed. psd_grms_g is the square root of the PSD area and should match grms_g. Band Grms are for the bands_hz ranges. Peaks are [frequency Hz, PSD g²/Hz], strongest first; the PSD is Welch, Hann window, 50 % overlap, 256 points at 400 Hz (1.56 Hz resolution).

{"v": 2, "dev": "kobra2neo", "ts": "2026-10-25T21:10:56.886Z", "gw": "unoq-aws-1", "seq": 2, "odr": 400, "window_s": 2,
 "sensors": {"table": {"n": 816, "odr": 400, "window_s": 2.04,
   "mean_g": [0.0001, 0.0, 1.0001], "grms_g": [0.07092, 0.03559, 0.00513], "peak_g": [0.10146, 0.04682, 0.01391],
   "crest": [1.43, 1.32, 2.71], "grms_total_g": 0.07951, "psd_grms_g": [0.07086, 0.03557, 0.00497],
   "bands_hz": [[0,10],[10,25],[25,50],[50,75],[75,100],[100,150],[150,200]],
   "bands_grms_g": {"x": [0.00028, 7e-05, 0.07085, 0.00062, 0.00049, 0.00062, 0.00044], "y": ["..."], "z": ["..."]},
   "peaks_hz_g2hz": {"x": [[45.31, 0.002026662], [104.69, 6.2e-08], [75.0, 5.5e-08]], "y": ["..."], "z": ["..."]},
   "df_hz": 1.5625, "nfft": 256},
  "toolhead": {"...": "same fields"}}}

Events (printer/kobra/event). type is printer_alert (a printer line containing error, thermal, runout, paused, resend and similar), print_started or print_finished.

{"v": 2, "dev": "kobra2neo", "ts": "...", "gw": "unoq-aws-1", "seq": 517, "type": "printer_alert", "at": "2026-10-25T21:30:02.114Z", "line": "Error:Printer halted. kill() called!"}
{"v": 2, "dev": "kobra2neo", "ts": "...", "gw": "unoq-aws-1", "seq": 518, "type": "print_started", "file": "part_PLA_1h5m.gcode"}
{"v": 2, "dev": "kobra2neo", "ts": "...", "gw": "unoq-aws-1", "seq": 519, "type": "print_finished", "print_time_s": 3911.0, "filament_mm": 5321.4}

Raw accelerometer (printer/kobra/accel_raw). One message per sensor per second. b64 is base64 of n samples, each three little-endian int16 values x, y, z; multiply by scale_g_per_lsb (3.9 mg) for g. Samples are evenly spaced at odr Hz. t_first_rx is when the first block reached the board's Linux side, accurate to about ±0.1 s. seq_first/seq_last count the 24-sample blocks from the sensor; gaps is how many blocks were lost inside this message.

{"v": 2, "dev": "kobra2neo", "ts": "2026-10-25T21:10:55.870Z", "gw": "unoq-aws-1", "sensor": "table", "odr": 400, "n": 432,
 "fmt": "int16le_xyz", "scale_g_per_lsb": 0.0039, "t_first_rx": "2026-10-25T21:10:54.815Z",
 "seq_first": 0, "seq_last": 17, "gaps": 0, "b64": "AAAAAAEBEQAMAP8AGQD4/wABFgD4/wEBCAAMAAAB..."}

Camera snapshot (printer/kobra/snapshot/<unix_ms>). The payload is a plain JPEG file, at most 120 kB and 1280 px wide. The topic's last level is the capture time in Unix milliseconds. There's no JSON envelope.

Setup and bed mesh (printer/kobra/setup, retained). It holds the printer's saved settings as read from M115/M503 (raw G-code strings plus a few parsed values, input shaping included) and the bed mesh. mesh is rows J0 to J4 (Y min first), each with 5 values in mm; mesh_info.complete says whether all rows arrived.

{"v": 2, "dev": "kobra2neo", "ts": "2026-10-25T21:10:56.887Z", "gw": "unoq-aws-1",
 "setup": {"steps": "M92 X80.00 Y80.00 Z400.00 E455.00", "hotend_pid": "M301 P14.42 I0.88 D59.12",
           "input_shaping": {"x": {"f": 55.77, "d": 0.15}, "y": {"f": 52.7, "d": 0.15}},
           "z_offset": -1.41, "leveling": true, "firmware": "Marlin bugfix-2.1.x (Aug 16 2023 15:33:11)", "...": "..."},
 "mesh": [[-0.682, -0.533, -0.363, -0.178, -0.013], [-0.399, -0.276, -0.118, 0.031, 0.173], "... 5 rows"],
 "mesh_info": {"complete": true, "tries": 1, "rows": 5, "rows_expected": 5, "row_idx": [0, 1, 2, 3, 4]}}

Online status (printer/kobra/status, retained). state is online or offline (the Last Will). printer is the printer link state, connected when healthy.

{"v": 2, "dev": "kobra2neo", "ts": "2026-10-25T21:10:56.882Z", "gw": "unoq-aws-1", "state": "online", "printer": "connected"}

## Delivery

Queued topics (telemetry, vibration, events) are written to a disk queue on the board first and deleted only after IoT Core acknowledges them. A network drop delays them and loses nothing; a 32-second outage in testing delivered 108 of 108 messages in order.

- seq is one counter shared by the three queued topics. It only ever increases, also across reboots. Store on (dev, seq) to drop duplicates, and treat a gap across all three topics together as lost data.

- Duplicates can happen. QoS 1 means at least once: a message whose acknowledgement was lost is sent again with the same seq.

- Replays keep their time. After an outage the backlog arrives in a burst, each message with its original ts. Order by ts (or seq), not by arrival time.

- Live-only topics (raw accelerometer, snapshots) are dropped while offline and never replayed. seq_first/seq_last on raw messages show any gap.

- Retained topics: status and setup are always the latest value, delivered at once to any new subscriber or rule test. status turns offline within about 45 s if the board loses power or network (Last Will, 30 s keepalive).

- Queue limit: the board keeps up to 200,000 queued messages, about 2 days of telemetry and vibration. Older ones are dropped first.

## Suggested rules (your call)

The board doesn't depend on any of these; it only needs the thing, certificate and policy. They're one sensible starting point for the dashboard.

| Data | Rule query | Suggested action |
| --- | --- | --- |
| Telemetry | SELECT * FROM 'printer/kobra/telemetry' | Time-series store (Timestream) or DynamoDB keyed on dev + seq |
| Vibration summary | SELECT * FROM 'printer/kobra/vibration' | Same store as telemetry |
| Raw accelerometer | SELECT * FROM 'printer/kobra/accel_raw' | Data Firehose to S3 in batches, for offline analysis |
| Snapshots | SELECT * FROM 'printer/kobra/snapshot/+' | S3, key kobra/snapshots/${topic(4)}.jpg; SELECT * passes the binary JPEG through unchanged |
| Printer alerts | SELECT * FROM 'printer/kobra/event' WHERE type = 'printer_alert' | SNS to email or phone |
| Online status | SELECT * FROM 'printer/kobra/status' | Latest-state table, or an alarm when state is offline |

## Board side

Everything the board sends is set in python/aws_config.json in the App Lab app; a change takes effect on the next app restart. The local web page shows the connection state and per-topic counts, and http://<board-ip>:7001/aws.json shows one example of every message exactly as last built.

| Setting | Default | What it does |
| --- | --- | --- |
| endpoint | to fill in | the -ats data endpoint from step 4 |
| client_id | kobra2neo-unoq | MQTT client ID; must match the policy |
| topic_prefix | printer/kobra | first two levels of every topic |
| telemetry_interval_s / vibration_interval_s | 2 / 2 | seconds between messages |
| raw_accel | true | false stops the raw stream (two thirds of all messages) |
| snapshot_interval_s | 30 | 0 stops snapshots |
| snapshot_max_kb / snapshot_max_width | 120 / 1280 | snapshots are shrunk to fit |
| dry_run | false | true builds every message without connecting; examples then appear on /aws.json |

Handover checklist

- ☐ Thing kobra2neo-unoq created, certificate active and attached

- ☐ Policy kobra2neo-unoq-policy attached to the certificate

- ☐ Endpoint address sent to Deepak

- ☐ AmazonRootCA1.pem, device.pem.crt, private.pem.key sent to Deepak privately

- ☐ MQTT test client in the IoT console subscribed to printer/kobra/# shows status online and telemetry every 2 s